Description

- Compact pot and pan washing machine that is front-loaded with door with double opening radius (90º and 180º) and rotating wash basket.

- High performing wash technology Aqua Power™ that operates with high flow and high pressure, which allows for reduced time, water, energy and detergent consumption while also minimising the need for soaking and pre-washing cookware.

- Operating panel at eye level. LED display and indicator lights showing program end. Preventive alerts: Change of wash water and time for service.

- Energy-saving mode if not used for 20 min.

- Unique QR code for online access to user manuals, servicing history, and troubleshooting guides.

- HACCP traceability via USB 2.0 port equipped with a waterproof lid. 200 latest wash program parameters can be exported.

- Built-in unit condensing steam using cold water. Water inlet: 5-20 °C - 53-125 ppm - 3-6 bar - 15 litres/min 
- ¾” male (DN 20). Extends the duration of the wash program up to 90 sec. Heat dissipation: Total: 1.7 kW - Sensible: 0.8 kW - Latent: 0.9 kW.

- Integrated dosing system with peristaltic pumps for liquid detergent and rinse aid. Detergent dosing sensor installed in the wash tank.

Included in Standard delivery

- Pro wash basket for Gastronorm pans and with integrated holders for pots, grids and trays.

- Utensils basket for ladles, whisks etc.

- Scraper for pre-scraping of pots and pans

- Installation manual, user manual and simplified user guide wall sticker.

Productivity

- 2 wash programs 2:30 min (+ 60 sec steam reduction) and 4:00 min (+ 60 sec steam reduction).

- Capacity per wash cycle: 6 GN 1/1 or GN 1/2 65 mm deep, 3 GN 1/1 200 mm deep and 3 GN 1/1 65 mm deep, or 2 GN 2/1 100 mm deep.

- Hourly productivity: 108 GN 1/1

Specifications and compliance

- Wash tank 108 litres. Heating element 9 kW. Wash temperature 65 °C. 2 Fixed wash arms. 12 polymer wash nozzles removable without tools. High-pressure pump 2.2 kW.

- Rinse tank 7.5 litres. Heating element 9 kW. Rinse temperature 85 °C. Rinse arms with 12 nozzles. 
Rinse pump 0.11 kW.

- Consumption of water per wash program: 6 litres.

- EN 1.4301 Stainless steel design: Frame, door, impeller, tanks, wash and rinse arms.

- Heat and noise insulation.

- Dimensions: W 1025 x D 957 x H 1736/2079 (± 25) mm. Weight: Net 257 kg - Gross 381 kg.

- Sound pressure level: 69.8 dB(A), ISO 11201

- Heat dissipation: Total 1.7 kW - Sensible 0.8 kW - Latent 0.9 kW.

- Norms and certification compliance IPX5 - CE - DIN 10534 - EN 1717 - RoHS.

Installation

- Electrical: 25 A – 11.5 kW - 3 AC 400-415 V - 50 Hz. Supplied with 2 metres long power cable to connect to an approved mains switch.

- Hot water softened inlet: 55-65 °C, 3-7 °dH (5-12 °fH) or 53-125 ppm, 1-6 bar, 15 litres/min – 3/4” male (DN 20).

- Drain: Ø 32 mm - 65 °C - 50 litres/min.

- Chemicals: 230 V built-in electrical connection box. Wash tank prepared for a detergent probe installation.

Factory fitted options

- Efficient steam condensing unit with heat recovery system. Saves up to 40% of the energy required to heat the rinse tank. Requires the high-power supply option. Connection to 5-20 °C cold water inlet. Extends the duration of the wash program up to 80 sec. Total height of the pot washing machine increased to 1964 (± 25) mm. Heat dissipation: Total: 1.3 kW - Sensible: 0.8 kW - Latent: 0.5 kW.

- Interface for energy optimisation. Relay terminal block that can be controlled by dry contact or 230 V.

- High-power supply version and other voltages - Cold or Hot water softened inlet:

  - 32 A - 16.5 kW - 3 AC 400-415 V - 50 Hz

  - 32 A - 16.5 kW 3 - AC 400-415 V - 60 Hz

- Standard power supply versions and other voltages - Hot-water softened inlet:

  - 25 A - 11.5 kW - 3 AC 400 V - 50 Hz

- Marine version: 2 level sensors in the wash tank to compensate for ship’s movement. Feet with floor anchoring system. Service access from the front.

Accessories

- Non-electric water softener for hot and cold water. Dual tank. 4 castors. Regeneration at 2 bar 24 litres/11 min. Salt capacity 18 kg. Dimensions: L 356 x W 356 x H 559 mm. Water inlet Ø 3/4” - 2-65 °C – 2.5-8.5 bar. Drain connection 0.375’’.

- Trolley for wash basket, 2 levels, 4 wheels with brakes. Dimensions: 800 x 870 mm.

- Multiflex holder for saucepans, small bowls, sieves. To be placed inside the wash basket. Capacity: up to 3-5 saucepans or small bowls, sieves.

- Flexible pot holder incl. 4 rubber straps and two hooks.

- Holder to store inserts and accessories not in use. Hung at either side of the pot washing machine.

- Set of 3 scrapers to eliminate food residue before loading pots or pans into the pot washing machine.